AMERICAN INTERNATIONAL UNIVERSITY-BANGLADESH (AIUB)

Department of CSE

Computer Aided Design and Drafting

Section: L

Experiment No: # 02

Name:  Khan, Md. Sazzad

Id:  22–49172-3

Supervised by:  MD. RAKIBUL ISLAM CHOWDHURY

Name of Experiment:  Familiarization with different toolbars and command of

AutoCAD software.

AutoCAD Comments:

- How To Save File In AutoCAD :

Type options >> Enter Key >> Open and Save >> Save as:  AutoCAD 2007 Drawing(*.dwg)>> OK

- How To Setup Units In AutoCad :

Type units>> Enter Key >> Type : Engineering, Decimal Degrees >> Precision :  0’ – 0.0”  , 0  >> Scale : Inches >> OK

- How To Draw A Line In AutoCAD:

Type L >> Press  Enter Key >> Left Click >> Put the Crosshair in the Right side and specify the Distance >> Press  Enter Key >>  Press  Esc(Exit from Line) .

- How To Regenerate In AutoCAD:

Type regen >> Press  Enter Key

- How To Dimension Style Setting In AutoCad :

- Type dimsty >> Enter Key >> modify  >> Primary Units  >> Unit format : Engineering >> Precision :  0’ – 0.0”

- Text >> Text Style :  … >>  modify  >> Font Name : Times New Roman >> Height : 10” >> Apply >> Close

- Text >> Text Height : 10”

- Symbol and arrows >>First :  Arch. Tick >>  Leader :  Arch. Tick >>ArrowSize: 6”

- Lines >> Extend Beyond  : 6”  >>Offset from Origin : 6” >> OK >> set Current >>           Close

- How To Calculate Line Length In AutoCAD :

For Linear Line : Type dimlinear >> Click On First Point >> Click On Second Point >> Place the dimension  line above the main line

For NonLinear Line : Type dimaligned >> Click On First Point >> Click On Second Point >> Place the dimension  line above the main line

- How To Calculate Angle Between Two Lines In AutoCAD :

Type dimangular  >> Click On First Point >> Click On Second Point >> Place the dimension  line Between two lines .

- How To do off set operation In AutoCAD :

Type O >> Enter Key >> Specify distance >> Enter Key >> Select Object >> Click on the Side where you want to off set.

- Type tr >> Enter Key >> Enter Key